# Supplementary material

Supporting tables and methodological detail for *Surrogate screening of trace metals in Bangladesh groundwater*. Every table is generated by the analysis code and read from disk; none is transcribed by hand. The pipeline that produces them is at https://github.com/Mosharrof8097/ground-water-.

## S1. Censoring read two ways

The paper argues that a detection limit read as though it were a measurement can manufacture structure as readily as it can hide it. Both cases below come from this analysis and are reported in full so the claim can be checked.

Table S1. Two cases in which reading a detection limit as a measurement creates structure that is not there.

| case | test | censoring in that column | reading the limit as a measurement | respecting the censoring flag | where the rest went | evidence |
| --- | --- | --- | --- | --- | --- | --- |
| Redox class: nitrate-reducing | NO3-N at or above 0.5 mg/L | 80 % of wells, limits 0.3 and 0.5 mg/L | 65 wells | 2 wells | 63 wells to mixed (anoxic) | median Fe 1.4 mg/L in the wells the naive test admits |
| Arsenic log-scale skill, 8 spatial blocks, REF tier | held-out log-scale R² above zero | 34 % of wells, limits 0.5 and 6.0 µg/L | R² = +0.227 | R² = +0.022 on measured wells only | not applicable | Wilcoxon p = 0.42 against zero |

In the first case the test is nitrate at or above 0.5 mg/L. Nitrate is censored in 80 % of these wells at limits of 0.3 and 0.5 mg/L, so a well reported as "< 0.5" carries the value 0.5 and passes. In the second the target is modelled on a log scale, where the lower tail carries disproportionate weight and the split between the two arsenic detection limits of 0.5 and 6.0 µg/L is itself learnable. Neither survives when the censoring flag is respected.

## S2. Full feasibility matrix

Every target and tier under random cross validation, with the model family selected inside the fold, the fold standard deviation, the Wilcoxon p value against a mean baseline and the Benjamini-Hochberg decision.

Table S2. Feasibility of every target and tier under random cross validation, with the model family selected inside the fold.

| target | tier | n | n features | r2 mean | r2 std | r2 baseline mean | delta r2 | rmse mean | spearman pooled | wilcoxon p | folds won vs baseline | dominant model | fdr significant | feasible |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AsTot | E1_meter | 271 | 4 | -0.07025 | 0.2946 | -0.1211 | 0.0509 | 281.3 | 0.4776 | 0.2454 | 15 | RandomForestRegressor | False | False |
| AsTot | E2_meter_kit | 271 | 6 | 0.03497 | 0.1486 | -0.1211 | 0.1561 | 274 | 0.6735 | 0.004411 | 17 | SVR | True | True |
| AsTot | E3_meter_kit_titration | 271 | 8 | 0.05764 | 0.1343 | -0.1211 | 0.1788 | 270.7 | 0.6898 | 0.0002148 | 21 | RandomForestRegressor | True | True |
| AsTot | E4_plus_well_record | 271 | 9 | 0.104 | 0.1407 | -0.1211 | 0.2251 | 264 | 0.7167 | 1.49e-07 | 24 | SVR | True | True |
| AsTot | REF_full_cheap | 271 | 14 | 0.06296 | 0.2961 | -0.1211 | 0.1841 | 267.2 | 0.7636 | 0.0008127 | 22 | RandomForestRegressor | True | True |
| Ba | E1_meter | 271 | 4 | 0.1404 | 0.1442 | -0.03484 | 0.1752 | 80.04 | 0.5107 | 3.183e-05 | 23 | RandomForestRegressor | True | True |
| Ba | E2_meter_kit | 271 | 6 | 0.2798 | 0.08983 | -0.03484 | 0.3146 | 73.27 | 0.5971 | 2.98e-08 | 25 | RandomForestRegressor | True | True |
| Ba | E3_meter_kit_titration | 271 | 8 | 0.3011 | 0.1645 | -0.03484 | 0.336 | 72.31 | 0.627 | 1.49e-07 | 23 | RandomForestRegressor | True | True |
| Ba | E4_plus_well_record | 271 | 9 | 0.5255 | 0.1103 | -0.03484 | 0.5603 | 59.64 | 0.7808 | 2.98e-08 | 25 | RandomForestRegressor | True | True |
| Ba | REF_full_cheap | 271 | 14 | 0.5261 | 0.102 | -0.03484 | 0.5609 | 59.76 | 0.7937 | 2.98e-08 | 25 | RandomForestRegressor | True | True |
| Mo | E1_meter | 271 | 4 | 0.08817 | 0.4862 | -0.0713 | 0.1595 | 2.527 | 0.5827 | 2.694e-05 | 24 | RandomForestRegressor | True | True |
| Mo | E2_meter_kit | 271 | 6 | 0.1249 | 0.3433 | -0.0713 | 0.1962 | 2.518 | 0.6428 | 2.271e-05 | 24 | RandomForestRegressor | True | True |
| Mo | E3_meter_kit_titration | 271 | 8 | 0.2138 | 0.2365 | -0.0713 | 0.2851 | 2.4 | 0.6577 | 2.98e-08 | 25 | RandomForestRegressor | True | True |
| Mo | E4_plus_well_record | 271 | 9 | 0.274 | 0.1896 | -0.0713 | 0.3453 | 2.328 | 0.7134 | 2.98e-08 | 25 | RandomForestRegressor | True | True |
| Mo | REF_full_cheap | 271 | 14 | 0.3611 | 0.1282 | -0.0713 | 0.4324 | 2.187 | 0.7672 | 2.98e-08 | 25 | RandomForestRegressor | True | True |
| Ni | E1_meter | 271 | 4 | 0.1693 | 0.09539 | -0.02193 | 0.1913 | 1.576 | 0.4998 | 2.98e-08 | 25 | RandomForestRegressor | True | True |
| Ni | E2_meter_kit | 271 | 6 | 0.234 | 0.1278 | -0.02193 | 0.2559 | 1.516 | 0.5874 | 5.96e-08 | 24 | RandomForestRegressor | True | True |
| Ni | E3_meter_kit_titration | 271 | 8 | 0.1466 | 0.1369 | -0.02193 | 0.1685 | 1.593 | 0.5599 | 7.451e-07 | 24 | Ridge | True | True |
| Ni | E4_plus_well_record | 271 | 9 | 0.1916 | 0.1704 | -0.02193 | 0.2135 | 1.549 | 0.5856 | 5.662e-07 | 24 | RandomForestRegressor | True | True |
| Ni | REF_full_cheap | 271 | 14 | 0.2877 | 0.1641 | -0.02193 | 0.3096 | 1.467 | 0.6509 | 2.98e-08 | 25 | SVR | True | True |
| Pb | E1_meter | 271 | 4 | -0.4893 | 1.05 | -1.164 | 0.6752 | 1.315 | 0.135 | 0.007986 | 14 | RandomForestRegressor | True | False |
| Pb | E2_meter_kit | 271 | 6 | -0.3118 | 1.097 | -1.164 | 0.8527 | 1.306 | 0.1285 | 0.0401 | 13 | ElasticNet | False | False |
| Pb | E3_meter_kit_titration | 271 | 8 | -0.1004 | 0.2025 | -1.164 | 1.064 | 1.296 | 0.1134 | 0.006777 | 14 | ElasticNet | True | False |
| Pb | E4_plus_well_record | 271 | 9 | -0.09884 | 0.2218 | -1.164 | 1.066 | 1.296 | 0.1291 | 0.006777 | 15 | ElasticNet | True | False |
| Pb | REF_full_cheap | 271 | 14 | -0.0118 | 0.1437 | -1.164 | 1.153 | 1.287 | 0.2307 | 0.00171 | 16 | SVR | True | False |
| U | E1_meter | 271 | 4 | -0.01898 | 0.05074 | -0.04262 | 0.02364 | 5.773 | 0.3039 | 0.2054 | 13 | RandomForestRegressor | False | False |
| U | E2_meter_kit | 271 | 6 | 0.3048 | 0.1444 | -0.04262 | 0.3474 | 4.71 | 0.8353 | 5.96e-08 | 24 | RandomForestRegressor | True | True |
| U | E3_meter_kit_titration | 271 | 8 | 0.3117 | 0.1722 | -0.04262 | 0.3543 | 4.711 | 0.8428 | 2.98e-08 | 25 | RandomForestRegressor | True | True |
| U | E4_plus_well_record | 271 | 9 | 0.3233 | 0.1816 | -0.04262 | 0.366 | 4.678 | 0.8402 | 2.98e-08 | 25 | RandomForestRegressor | True | True |
| U | REF_full_cheap | 271 | 14 | 0.3736 | 0.1777 | -0.04262 | 0.4163 | 4.43 | 0.8743 | 1.49e-07 | 23 | RandomForestRegressor | True | True |
| Zn | E1_meter | 260 | 4 | -0.0321 | 0.06661 | -0.02039 | -0.01171 | 22.47 | 0.2516 | 0.8564 | 8 | HuberRegressor | False | False |
| Zn | E2_meter_kit | 260 | 6 | -0.02854 | 0.08391 | -0.02039 | -0.008156 | 22.48 | 0.2447 | 0.7288 | 9 | HuberRegressor | False | False |
| Zn | E3_meter_kit_titration | 260 | 8 | -0.004467 | 0.1036 | -0.02039 | 0.01592 | 22.27 | 0.2544 | 0.5837 | 10 | HuberRegressor | False | False |
| Zn | E4_plus_well_record | 260 | 9 | -0.0146 | 0.09864 | -0.02039 | 0.005788 | 22.36 | 0.255 | 0.7791 | 8 | ElasticNet | False | False |
| Zn | REF_full_cheap | 260 | 14 | -0.05014 | 0.08496 | -0.02039 | -0.02976 | 22.67 | 0.2593 | 0.9178 | 7 | RandomForestRegressor | False | False |

## S3. Random against spatially blocked cross validation

All seven modelled targets, five tiers and two block counts, with the optimism gap each combination carries.

Table S3. Random against spatially blocked cross validation for every target, tier and block count.

| target | tier | n blocks | random r2 | spatial r2 | spatial r2 std | spatial r2 log | optimism gap | wilcoxon p | n folds | random feasible | spatial feasible |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AsTot | E1_meter | 8 | -0.0702 | -0.0332 | 0.1467 | -0.1748 | -0.0371 | 0.04121 | 25 | False | False |
| AsTot | E1_meter | 16 | -0.0702 | 0.078 | 0.2203 | 0.0303 | -0.1483 | 0.0003 | 25 | False | True |
| AsTot | E2_meter_kit | 8 | 0.035 | -0.2296 | 1.068 | 0.1178 | 0.2646 | 0.0034 | 25 | True | False |
| AsTot | E2_meter_kit | 16 | 0.035 | -0.1608 | 0.7674 | 0.1929 | 0.1958 | 0.00641 | 25 | True | False |
| AsTot | E3_meter_kit_titration | 8 | 0.0576 | -0.1057 | 0.6013 | 0.1371 | 0.1633 | 0.01227 | 25 | True | False |
| AsTot | E3_meter_kit_titration | 16 | 0.0576 | 0.0384 | 0.3172 | 0.2513 | 0.0192 | 0.00011 | 25 | True | True |
| AsTot | E4_plus_well_record | 8 | 0.104 | -0.0652 | 0.4637 | 0.1617 | 0.1692 | 0.00858 | 25 | True | False |
| AsTot | E4_plus_well_record | 16 | 0.104 | 0.0783 | 0.2952 | 0.2693 | 0.0257 | 4e-05 | 25 | True | True |
| AsTot | REF_full_cheap | 8 | 0.063 | -0.038 | 0.6508 | 0.2265 | 0.1009 | 0.00049 | 25 | True | False |
| AsTot | REF_full_cheap | 16 | 0.063 | 0.0056 | 0.7948 | 0.3426 | 0.0573 | 0.00078 | 25 | True | True |
| Ni | E1_meter | 8 | 0.1693 | -0.3192 | 0.4259 | -0.3268 | 0.4885 | 0.1377 | 25 | True | False |
| Ni | E1_meter | 16 | 0.1693 | -0.3302 | 0.769 | -0.2791 | 0.4995 | 0.2814 | 25 | True | False |
| Ni | E2_meter_kit | 8 | 0.234 | -0.2615 | 0.3497 | -0.235 | 0.4955 | 0.06408 | 25 | True | False |
| Ni | E2_meter_kit | 16 | 0.234 | -0.2225 | 0.4448 | -0.1445 | 0.4565 | 0.00641 | 25 | True | False |
| Ni | E3_meter_kit_titration | 8 | 0.1466 | -0.3684 | 0.3843 | -0.3235 | 0.515 | 0.3784 | 25 | True | False |
| Ni | E3_meter_kit_titration | 16 | 0.1466 | -0.2759 | 0.4134 | -0.1765 | 0.4225 | 0.0871 | 25 | True | False |
| Ni | E4_plus_well_record | 8 | 0.1916 | -0.28 | 0.3318 | -0.2405 | 0.4716 | 0.04364 | 25 | True | False |
| Ni | E4_plus_well_record | 16 | 0.1916 | -0.2202 | 0.3618 | -0.1239 | 0.4118 | 0.00123 | 25 | True | False |
| Ni | REF_full_cheap | 8 | 0.2877 | -0.2039 | 0.3167 | -0.1698 | 0.4916 | 0.00102 | 25 | True | False |
| Ni | REF_full_cheap | 16 | 0.2877 | -0.1127 | 0.3449 | -0.0276 | 0.4004 | 1e-05 | 25 | True | False |
| Pb | E1_meter | 8 | -0.4893 | -1.628 | 3.461 | -1.03 | 1.138 | 0.8165 | 25 | False | False |
| Pb | E1_meter | 16 | -0.4893 | -0.4374 | 0.7878 | -0.4545 | -0.0519 | 0.1379 | 25 | False | False |
| Pb | E2_meter_kit | 8 | -0.3118 | -0.1449 | 0.1875 | -0.2098 | -0.1669 | 0.06077 | 25 | False | False |
| Pb | E2_meter_kit | 16 | -0.3118 | -0.6454 | 1.444 | -0.4637 | 0.3336 | 0.3482 | 25 | False | False |
| Pb | E3_meter_kit_titration | 8 | -0.1004 | -0.1752 | 0.2753 | -0.2435 | 0.0748 | 0.1154 | 25 | False | False |
| Pb | E3_meter_kit_titration | 16 | -0.1004 | -0.1998 | 0.3561 | -0.2599 | 0.0993 | 0.06091 | 25 | False | False |
| Pb | E4_plus_well_record | 8 | -0.0988 | -0.1568 | 0.2563 | -0.2131 | 0.058 | 0.06077 | 25 | False | False |
| Pb | E4_plus_well_record | 16 | -0.0988 | -0.1708 | 0.3611 | -0.2175 | 0.072 | 0.06091 | 25 | False | False |
| Pb | REF_full_cheap | 8 | -0.0118 | -0.157 | 0.2272 | -0.2255 | 0.1452 | 0.02709 | 25 | False | False |
| Pb | REF_full_cheap | 16 | -0.0118 | -0.1751 | 0.2608 | -0.2445 | 0.1633 | 0.03073 | 25 | False | False |
| Zn | E1_meter | 8 | -0.0321 | -0.0773 | 0.1277 | -0.1177 | 0.0452 | 0.6519 | 25 | False | False |
| Zn | E1_meter | 16 | -0.0321 | -0.0826 | 0.1422 | -0.0743 | 0.0505 | 0.5 | 25 | False | False |
| Zn | E2_meter_kit | 8 | -0.0285 | -0.0955 | 0.0932 | -0.1524 | 0.067 | 0.7623 | 25 | False | False |
| Zn | E2_meter_kit | 16 | -0.0285 | -0.0912 | 0.1527 | -0.0792 | 0.0626 | 0.6444 | 25 | False | False |
| Zn | E3_meter_kit_titration | 8 | -0.0045 | -0.0363 | 0.1062 | -0.078 | 0.0318 | 0.1102 | 25 | False | False |
| Zn | E3_meter_kit_titration | 16 | -0.0045 | -0.0368 | 0.1548 | -0.0222 | 0.0323 | 0.1763 | 25 | False | False |
| Zn | E4_plus_well_record | 8 | -0.0146 | -0.061 | 0.1133 | -0.1069 | 0.0464 | 0.1764 | 25 | False | False |
| Zn | E4_plus_well_record | 16 | -0.0146 | -0.0451 | 0.1528 | -0.0263 | 0.0305 | 0.2131 | 25 | False | False |
| Zn | REF_full_cheap | 8 | -0.0501 | -1.681 | 3.101 | -0.3126 | 1.631 | 0.8501 | 25 | False | False |
| Zn | REF_full_cheap | 16 | -0.0501 | -3.14 | 8.34 | -0.2732 | 3.09 | 0.9817 | 25 | False | False |
| U | E1_meter | 8 | -0.019 | -0.1676 | 0.4775 | -0.2585 | 0.1486 | 0.02709 | 25 | False | False |
| U | E1_meter | 16 | -0.019 | -0.0597 | 0.0836 | -0.1081 | 0.0407 | 0.8679 | 25 | False | False |
| U | E2_meter_kit | 8 | 0.3048 | 0.1269 | 0.5099 | 0.4342 | 0.1779 | 1e-05 | 25 | True | True |
| U | E2_meter_kit | 16 | 0.3048 | 0.1936 | 0.1653 | 0.4165 | 0.1112 | 1e-05 | 25 | True | True |
| U | E3_meter_kit_titration | 8 | 0.3117 | 0.1998 | 0.231 | 0.4627 | 0.1119 | 1e-05 | 25 | True | True |
| U | E3_meter_kit_titration | 16 | 0.3117 | 0.2284 | 0.0934 | 0.4573 | 0.0833 | 1e-05 | 25 | True | True |
| U | E4_plus_well_record | 8 | 0.3233 | 0.2219 | 0.2928 | 0.4644 | 0.1015 | 1e-05 | 25 | True | True |
| U | E4_plus_well_record | 16 | 0.3233 | 0.2507 | 0.1101 | 0.4656 | 0.0727 | 1e-05 | 25 | True | True |
| U | REF_full_cheap | 8 | 0.3736 | 0.2656 | 0.356 | 0.5592 | 0.1081 | 1e-05 | 25 | True | True |
| U | REF_full_cheap | 16 | 0.3736 | 0.3538 | 0.1794 | 0.5932 | 0.0199 | 1e-05 | 25 | True | True |
| Mo | E1_meter | 8 | 0.0882 | 0.1229 | 0.1772 | 0.2044 | -0.0348 | 6e-05 | 25 | True | True |
| Mo | E1_meter | 16 | 0.0882 | 0.0887 | 0.24 | 0.1824 | -0.0005 | 1e-05 | 25 | True | True |
| Mo | E2_meter_kit | 8 | 0.1249 | 0.1291 | 0.1418 | 0.1934 | -0.0041 | 1e-05 | 25 | True | True |
| Mo | E2_meter_kit | 16 | 0.1249 | 0.1355 | 0.1488 | 0.2513 | -0.0106 | 1e-05 | 25 | True | True |
| Mo | E3_meter_kit_titration | 8 | 0.2138 | 0.1805 | 0.1013 | 0.2649 | 0.0332 | 1e-05 | 25 | True | True |
| Mo | E3_meter_kit_titration | 16 | 0.2138 | 0.1825 | 0.1249 | 0.2724 | 0.0313 | 1e-05 | 25 | True | True |
| Mo | E4_plus_well_record | 8 | 0.274 | 0.232 | 0.1313 | 0.2969 | 0.042 | 1e-05 | 25 | True | True |
| Mo | E4_plus_well_record | 16 | 0.274 | 0.2243 | 0.1607 | 0.3284 | 0.0497 | 1e-05 | 25 | True | True |
| Mo | REF_full_cheap | 8 | 0.3611 | 0.3258 | 0.1151 | 0.3704 | 0.0353 | 1e-05 | 25 | True | True |
| Mo | REF_full_cheap | 16 | 0.3611 | 0.2831 | 0.1536 | 0.3923 | 0.078 | 1e-05 | 25 | True | True |
| Ba | E1_meter | 8 | 0.1404 | -0.6066 | 0.755 | -1.403 | 0.747 | 0.04886 | 25 | True | False |
| Ba | E1_meter | 16 | 0.1404 | -0.2913 | 0.5707 | -0.7008 | 0.4317 | 0.1697 | 25 | True | False |
| Ba | E2_meter_kit | 8 | 0.2798 | -0.3547 | 0.6882 | -1.184 | 0.6345 | 0.03456 | 25 | True | False |
| Ba | E2_meter_kit | 16 | 0.2798 | -0.237 | 0.7493 | -0.6142 | 0.5168 | 0.1006 | 25 | True | False |
| Ba | E3_meter_kit_titration | 8 | 0.3011 | -0.529 | 0.8956 | -1.19 | 0.8301 | 0.09128 | 25 | True | False |
| Ba | E3_meter_kit_titration | 16 | 0.3011 | -0.2083 | 0.5114 | -0.6085 | 0.5094 | 0.1006 | 25 | True | False |
| Ba | E4_plus_well_record | 8 | 0.5255 | -0.0488 | 0.322 | -0.1536 | 0.5742 | 1e-05 | 25 | True | False |
| Ba | E4_plus_well_record | 16 | 0.5255 | 0.2374 | 0.2331 | 0.2401 | 0.2881 | 1e-05 | 25 | True | True |
| Ba | REF_full_cheap | 8 | 0.5261 | 0.0061 | 0.3215 | -0.0388 | 0.52 | 1e-05 | 25 | True | True |
| Ba | REF_full_cheap | 16 | 0.5261 | 0.259 | 0.2497 | 0.2891 | 0.2671 | 1e-05 | 25 | True | True |

## S4. Descriptive statistics

All predictors, targets and geochemical determinands, with censoring counts, the detection limits in use and the distribution.

Table S4. Descriptive statistics for all predictors, targets and geochemistry, with censoring and detection limits.

| variable | role | unit | n valid | n censored | pct censored | detection limits | min | p25 | median | p75 | max | mean | sd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cl | predictor | mg/L | 269 | 0 | 0 |  | 1 | 5 | 14.2 | 55.5 | 1550 | 78.08 | 210 |
| Eh | predictor | mV | 269 | 0 | 0 |  | -105 | 42 | 82 | 119 | 341 | 85.48 | 71.14 |
| HCO3 | predictor | mg/L | 271 | 0 | 0 |  | 125 | 377.5 | 474 | 577 | 1140 | 473.9 | 162.1 |
| SEC | predictor | uS/cm | 271 | 0 | 0 |  | 96.8 | 591 | 742 | 949.5 | 4640 | 864.8 | 560.6 |
| T | predictor | deg C | 271 | 0 | 0 |  | 24 | 25.9 | 26.5 | 27 | 31.1 | 26.48 | 0.9913 |
| WELL_DEPTH | predictor | m | 261 | 0 | 0 |  | 3.6 | 14 | 28 | 38.7 | 375 | 47.05 | 65.51 |
| pH | predictor | pH units | 256 | 0 | 0 |  | 5.9 | 6.84 | 7.01 | 7.143 | 8.39 | 6.992 | 0.2401 |
| Fe | predictor | mg/L | 271 | 1 | 0.4 | 0.006 | 0.006 | 0.267 | 1.43 | 4.645 | 24.8 | 2.979 | 3.853 |
| Mn | predictor | mg/L | 271 | 1 | 0.4 | 0.002 | 0.002 | 0.1985 | 0.543 | 0.9865 | 4.36 | 0.6734 | 0.6209 |
| DOC | predictor | mg/L | 258 | 9 | 3.5 | 0.1; 1 | 0 | 1 | 1.7 | 3.075 | 14 | 2.461 | 2.384 |
| SO4 | predictor | mg/L | 271 | 57 | 21 | 0.2 | 0.2 | 0.3 | 1.1 | 10.1 | 335 | 12.85 | 34.72 |
| NH4-N | predictor | mg/L | 271 | 87 | 32.1 | 0.01; 0.02 | 0.01 | 0.02 | 0.38 | 1.535 | 17.8 | 1.339 | 2.535 |
| diss O2 | predictor | mg/L | 268 | 201 | 75 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 6.8 | 0.2813 | 0.6419 |
| NO3-N | predictor | mg/L | 271 | 217 | 80.1 | 0.01; 0.3; 0.5 | 0.01 | 0.3 | 0.3 | 0.5 | 68 | 1.189 | 5.093 |
| Ba | target | ug/L | 271 | 0 | 0 |  | 5 | 55 | 114 | 169.5 | 657 | 121.9 | 89 |
| Zn | target | ug/L | 260 | 1 | 0.4 | 1 | 1 | 4.85 | 8 | 20 | 301 | 16.58 | 24.62 |
| Ni | target | ug/L | 271 | 3 | 1.1 | 0.4 | 0.4 | 1.75 | 2.4 | 3.3 | 18.8 | 2.846 | 1.797 |
| Mo | target | ug/L | 271 | 20 | 7.4 | 0.1 | 0.1 | 0.4 | 1.1 | 2.2 | 20.5 | 2.027 | 2.888 |
| Pb | target | ug/L | 271 | 34 | 12.5 | 0.01; 0.05 | 0.01 | 0.06 | 0.1 | 0.2 | 28.6 | 0.3515 | 1.914 |
| U | target | ug/L | 271 | 62 | 22.9 | 0.007; 0.01 | 0.007 | 0.01 | 0.42 | 2.345 | 47.1 | 2.647 | 6.007 |
| AsTot | target | ug/L | 271 | 92 | 33.9 | 0.5; 6 | 0.5 | 6 | 22 | 109 | 2400 | 125.3 | 298 |
| Sb | target | ug/L | 271 | 126 | 46.5 | 0.02; 0.1 | 0.02 | 0.02 | 0.03 | 0.07 | 1.18 | 0.0576 | 0.0907 |
| AsIII | target | ug/L | 270 | 140 | 51.9 | 0.5; 4; 6 | 0.5 | 6 | 6 | 53.75 | 2540 | 71.86 | 234.3 |
| Cu | target | ug/L | 271 | 220 | 81.2 | 1 | 0.03 | 1 | 1 | 1 | 19.3 | 1.075 | 1.238 |
| Cd | target | ug/L | 271 | 228 | 84.1 | 0.02; 0.06 | 0.02 | 0.02 | 0.02 | 0.02 | 0.19 | 0.0285 | 0.0218 |
| Cr | target | ug/L | 271 | 263 | 97 | 0.5; 0.6 | 0.5 | 0.5 | 0.5 | 0.5 | 1.3 | 0.5251 | 0.102 |
| Se | target | ug/L | 19 | 19 | 100 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0 |
| Ca | geochemistry | mg/L | 271 | 0 | 0 |  | 16.6 | 57.15 | 87.1 | 116 | 259 | 89.66 | 43.06 |
| F | geochemistry | mg/L | 271 | 0 | 0 |  | 0.01 | 0.12 | 0.18 | 0.295 | 1.28 | 0.2262 | 0.1543 |
| K | geochemistry | mg/L | 271 | 0 | 0 |  | 0.5 | 3.65 | 4.8 | 6.8 | 41.3 | 6.22 | 5.479 |
| Mg | geochemistry | mg/L | 271 | 0 | 0 |  | 6.34 | 22.55 | 28.6 | 39.45 | 154 | 33.66 | 20.77 |
| Na | geochemistry | mg/L | 271 | 0 | 0 |  | 5.1 | 16.2 | 27.1 | 61.75 | 1090 | 68.55 | 131.1 |
| Si | geochemistry | mg/L | 271 | 0 | 0 |  | 6.15 | 13 | 15.3 | 18.8 | 41.9 | 16.34 | 5.472 |
| Sr | geochemistry | mg/L | 271 | 0 | 0 |  | 0.0968 | 0.2715 | 0.353 | 0.448 | 1.4 | 0.3825 | 0.1828 |
| I | geochemistry | ug/L | 271 | 2 | 0.7 | 0.8 | 0.5 | 6.4 | 19.3 | 56.05 | 1240 | 59.49 | 139.6 |
| Br | geochemistry | mg/L | 270 | 13 | 4.8 | 0.001; 0.02 | 0.001 | 0.0243 | 0.0555 | 0.13 | 5.6 | 0.291 | 0.8078 |

## S5. Conformal coverage by target and tier

Held-out coverage over 100 disjoint partitions, with interval width and the calibrated quantile.

Table S5. Held-out split conformal coverage and interval width for each feasible target and tier.

| target | tier | nominal coverage | held out coverage mean | held out coverage std | mean interval width | median q hat |
| --- | --- | --- | --- | --- | --- | --- |
| AsTot | E2_meter_kit | 0.9 | 0.9042 | 0.0552 | 314.7 | 221.2 |
| AsTot | E3_meter_kit_titration | 0.9 | 0.9043 | 0.0501 | 310.9 | 211 |
| AsTot | E4_plus_well_record | 0.9 | 0.9099 | 0.0514 | 318.5 | 214.8 |
| AsTot | REF_full_cheap | 0.9 | 0.9042 | 0.0554 | 298.7 | 210.5 |
| Ni | E1_meter | 0.9 | 0.9064 | 0.0526 | 4.913 | 2.645 |
| Ni | E2_meter_kit | 0.9 | 0.9025 | 0.0538 | 4.598 | 2.413 |
| Ni | E3_meter_kit_titration | 0.9 | 0.8981 | 0.0583 | 4.577 | 2.422 |
| Ni | E4_plus_well_record | 0.9 | 0.9031 | 0.0543 | 4.357 | 2.28 |
| Ni | REF_full_cheap | 0.9 | 0.9012 | 0.0522 | 4.132 | 2.084 |
| U | E2_meter_kit | 0.9 | 0.9104 | 0.0565 | 7.939 | 6.447 |
| U | E3_meter_kit_titration | 0.9 | 0.9119 | 0.0554 | 7.853 | 6.146 |
| U | E4_plus_well_record | 0.9 | 0.9115 | 0.055 | 7.726 | 6.022 |
| U | REF_full_cheap | 0.9 | 0.9131 | 0.0513 | 7.19 | 5.649 |
| Mo | E1_meter | 0.9 | 0.911 | 0.0461 | 5.348 | 3.826 |
| Mo | E2_meter_kit | 0.9 | 0.9119 | 0.0515 | 5.349 | 3.748 |
| Mo | E3_meter_kit_titration | 0.9 | 0.9155 | 0.0529 | 5.221 | 3.712 |
| Mo | E4_plus_well_record | 0.9 | 0.917 | 0.0521 | 5.146 | 3.575 |
| Mo | REF_full_cheap | 0.9 | 0.907 | 0.0538 | 4.403 | 2.865 |
| Ba | E1_meter | 0.9 | 0.911 | 0.0478 | 219.2 | 125.8 |
| Ba | E2_meter_kit | 0.9 | 0.9128 | 0.0501 | 212.2 | 123.1 |
| Ba | E3_meter_kit_titration | 0.9 | 0.9116 | 0.0483 | 207.4 | 119.7 |
| Ba | E4_plus_well_record | 0.9 | 0.911 | 0.0517 | 179.7 | 99.49 |
| Ba | REF_full_cheap | 0.9 | 0.9149 | 0.0518 | 183.3 | 103 |

## S6. Permutation importance

Every modelled metal, with redox indicators flagged. Importance is the mean drop in log-scale R² on held-out folds and is therefore not on the same scale as the coefficients of determination reported in the main text.

Table S6. Permutation feature importance for every modelled metal, measured as drop in log-scale R² on held-out folds.

| target | feature | mean r2 drop | sd | share | redox proxy | spatially robust target |
| --- | --- | --- | --- | --- | --- | --- |
| AsTot | Fe | 0.3928 | 0.1121 | 0.3665 | True | True |
| AsTot | pH | 0.3365 | 0.1263 | 0.314 | False | True |
| AsTot | WELL_DEPTH | 0.1016 | 0.04504 | 0.0948 | False | True |
| AsTot | Mn | 0.09487 | 0.06339 | 0.0885 | True | True |
| AsTot | HCO3 | 0.05827 | 0.05081 | 0.0544 | False | True |
| AsTot | T | 0.05039 | 0.02452 | 0.047 | False | True |
| AsTot | SEC | 0.02347 | 0.03518 | 0.0219 | False | True |
| AsTot | Cl | 0.00984 | 0.0209 | 0.0092 | False | True |
| AsTot | Eh | 0.00394 | 0.03224 | 0.0037 | True | True |
| Ni | Mn | 0.1343 | 0.067 | 0.3347 | True | False |
| Ni | SEC | 0.1093 | 0.101 | 0.2725 | False | False |
| Ni | HCO3 | 0.06559 | 0.05437 | 0.1635 | False | False |
| Ni | Eh | 0.02791 | 0.03059 | 0.0696 | True | False |
| Ni | WELL_DEPTH | 0.02281 | 0.03775 | 0.0569 | False | False |
| Ni | pH | 0.02146 | 0.02039 | 0.0535 | False | False |
| Ni | Cl | 0.00969 | 0.02856 | 0.0242 | False | False |
| Ni | Fe | 0.00566 | 0.01198 | 0.0141 | True | False |
| Ni | T | 0.00452 | 0.0141 | 0.0113 | False | False |
| Pb | HCO3 | 0.03872 | 0.07385 | 0.287 | False | False |
| Pb | WELL_DEPTH | 0.02653 | 0.06662 | 0.1966 | False | False |
| Pb | Mn | 0.01979 | 0.03145 | 0.1467 | True | False |
| Pb | Eh | 0.01846 | 0.03319 | 0.1368 | True | False |
| Pb | SEC | 0.0125 | 0.02383 | 0.0926 | False | False |
| Pb | pH | 0.00764 | 0.01433 | 0.0566 | False | False |
| Pb | Cl | 0.0065 | 0.02097 | 0.0482 | False | False |
| Pb | Fe | 0.00478 | 0.03543 | 0.0354 | True | False |
| Pb | T | -0.00548 | 0.00873 | 0 | False | False |
| Zn | Cl | 0.07616 | 0.1036 | 0.3159 | False | False |
| Zn | HCO3 | 0.04449 | 0.06564 | 0.1845 | False | False |
| Zn | Eh | 0.03928 | 0.01704 | 0.1629 | True | False |
| Zn | SEC | 0.03892 | 0.05833 | 0.1614 | False | False |
| Zn | T | 0.03635 | 0.03996 | 0.1508 | False | False |
| Zn | Fe | 0.00588 | 0.00913 | 0.0244 | True | False |
| Zn | WELL_DEPTH | -0.00137 | 0.03081 | 0 | False | False |
| Zn | Mn | -0.00234 | 0.01465 | 0 | True | False |
| Zn | pH | -0.0246 | 0.03423 | 0 | False | False |
| U | Fe | 0.9825 | 0.1148 | 0.7925 | True | True |
| U | HCO3 | 0.121 | 0.04894 | 0.0976 | False | True |
| U | SEC | 0.06594 | 0.04361 | 0.0532 | False | True |
| U | Mn | 0.04471 | 0.02796 | 0.0361 | True | True |
| U | pH | 0.01505 | 0.00804 | 0.0121 | False | True |
| U | Eh | 0.00476 | 0.01148 | 0.0038 | True | True |
| U | WELL_DEPTH | 0.00435 | 0.01706 | 0.0035 | False | True |
| U | Cl | 0.00148 | 0.00962 | 0.0012 | False | True |
| U | T | -0.00215 | 0.00414 | 0 | False | True |
| Mo | pH | 0.466 | 0.09831 | 0.4942 | False | True |
| Mo | HCO3 | 0.2081 | 0.08596 | 0.2207 | False | True |
| Mo | WELL_DEPTH | 0.09546 | 0.06776 | 0.1012 | False | True |
| Mo | Fe | 0.07389 | 0.03121 | 0.0784 | True | True |
| Mo | T | 0.03077 | 0.03836 | 0.0326 | False | True |
| Mo | Mn | 0.02882 | 0.01418 | 0.0306 | True | True |
| Mo | Eh | 0.02495 | 0.02875 | 0.0265 | True | True |
| Mo | SEC | 0.00798 | 0.01495 | 0.0085 | False | True |
| Mo | Cl | 0.00689 | 0.01779 | 0.0073 | False | True |
| Ba | WELL_DEPTH | 0.8118 | 0.129 | 0.6762 | False | True |
| Ba | SEC | 0.171 | 0.05314 | 0.1424 | False | True |
| Ba | Fe | 0.09428 | 0.0275 | 0.0785 | True | True |
| Ba | pH | 0.05036 | 0.01995 | 0.0419 | False | True |
| Ba | HCO3 | 0.03197 | 0.02474 | 0.0266 | False | True |
| Ba | Mn | 0.02697 | 0.01349 | 0.0225 | True | True |
| Ba | Eh | 0.00815 | 0.00683 | 0.0068 | True | True |
| Ba | T | 0.00381 | 0.00599 | 0.0032 | False | True |
| Ba | Cl | 0.00226 | 0.00418 | 0.0019 | False | True |

## S7. Learning curves

Held-out R², mean, median and interquartile range against training set size. The mean is reported for completeness only: at the smallest training size a single blown fold takes molybdenum to −12.4 while its median stays at +0.16.

Table S7. Learning curve results, held-out R² against training set size.

| target | fraction of pool | n train | n test | r2 mean | r2 sd | r2 median | r2 q1 | r2 q3 | r2 log mean | r2 log median |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AsTot | 0.25 | 51 | 68 | -0.08185 | 0.6296 | -0.0103 | -0.0714 | 0.03319 | 0.3228 | 0.3228 |
| AsTot | 0.4 | 81 | 68 | 0.03344 | 0.1489 | 0.02088 | -0.05534 | 0.1422 | 0.3712 | 0.3666 |
| AsTot | 0.55 | 112 | 68 | -5.023 | 27.46 | 0.005621 | -0.08115 | 0.08137 | 0.37 | 0.3981 |
| AsTot | 0.7 | 142 | 68 | 0.07435 | 0.1129 | 0.06567 | -0.0324 | 0.1599 | 0.4716 | 0.4758 |
| AsTot | 0.85 | 173 | 68 | 0.07893 | 0.1214 | 0.03731 | 0.002858 | 0.09383 | 0.4987 | 0.5042 |
| AsTot | 1 | 203 | 68 | 0.0733 | 0.1167 | 0.0718 | -0.003272 | 0.1545 | 0.492 | 0.4984 |
| U | 0.25 | 51 | 68 | 0.1795 | 0.129 | 0.1568 | 0.07011 | 0.2797 | 0.4137 | 0.4251 |
| U | 0.4 | 81 | 68 | 0.0929 | 0.8173 | 0.2246 | 0.1413 | 0.2852 | 0.4714 | 0.4717 |
| U | 0.55 | 112 | 68 | 0.2922 | 0.1209 | 0.2752 | 0.2117 | 0.357 | 0.5646 | 0.5769 |
| U | 0.7 | 142 | 68 | 0.2749 | 0.1174 | 0.2869 | 0.2124 | 0.3419 | 0.5266 | 0.5463 |
| U | 0.85 | 173 | 68 | 0.3299 | 0.1352 | 0.36 | 0.2364 | 0.4225 | 0.5962 | 0.606 |
| U | 1 | 203 | 68 | 0.3336 | 0.1087 | 0.3081 | 0.2622 | 0.4286 | 0.5906 | 0.5997 |
| Mo | 0.25 | 51 | 68 | -12.44 | 78.49 | 0.1641 | 0.05807 | 0.2533 | 0.2322 | 0.2722 |
| Mo | 0.4 | 81 | 68 | 0.1966 | 0.1228 | 0.1932 | 0.1529 | 0.2623 | 0.3463 | 0.3534 |
| Mo | 0.55 | 112 | 68 | 0.1999 | 0.2013 | 0.197 | 0.154 | 0.2964 | 0.3505 | 0.3638 |
| Mo | 0.7 | 142 | 68 | 0.2334 | 0.1068 | 0.2378 | 0.1607 | 0.281 | 0.4233 | 0.4276 |
| Mo | 0.85 | 173 | 68 | 0.2984 | 0.1113 | 0.2875 | 0.2194 | 0.3731 | 0.4598 | 0.4672 |
| Mo | 1 | 203 | 68 | 0.3558 | 0.1171 | 0.3465 | 0.2646 | 0.4061 | 0.5046 | 0.5322 |
| Ba | 0.25 | 51 | 68 | -0.1584 | 2.604 | 0.3049 | 0.2013 | 0.4038 | 0.3968 | 0.4393 |
| Ba | 0.4 | 81 | 68 | 0.4333 | 0.1464 | 0.4481 | 0.3219 | 0.5538 | 0.5319 | 0.5372 |
| Ba | 0.55 | 112 | 68 | 0.4707 | 0.1074 | 0.4717 | 0.3949 | 0.5653 | 0.5506 | 0.5671 |
| Ba | 0.7 | 142 | 68 | 0.481 | 0.1553 | 0.5038 | 0.4129 | 0.5835 | 0.5976 | 0.6142 |
| Ba | 0.85 | 173 | 68 | 0.5067 | 0.1099 | 0.5312 | 0.4368 | 0.5814 | 0.5925 | 0.5852 |
| Ba | 1 | 203 | 68 | 0.4942 | 0.1138 | 0.5093 | 0.4106 | 0.557 | 0.6133 | 0.6443 |

## S8. Arsenic log-scale skill, with and without the censored wells

Identical spatially blocked folds scored twice, on all test wells and on measured test wells only, with the random-fold comparison that gives the optimism gap on the log scale.

Table S8. Arsenic log-scale skill scored on all test wells and on measured test wells only, under identical folds.

| target | tier | n blocks | random r2 | random r2 log | random r2 log measured | spatial r2 | spatial r2 log | spatial r2 log measured | log optimism gap | wilcoxon p log measured |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AsTot | E1_meter | 8 | -0.0702 | 0.1867 | -0.1621 | -0.0332 | -0.1748 | -0.3817 | 0.3614 | 0.9984 |
| AsTot | E1_meter | 16 | -0.0702 | 0.1867 | -0.1621 | 0.078 | 0.0303 | -0.2373 | 0.1564 | 0.9587 |
| AsTot | E2_meter_kit | 8 | 0.035 | 0.3928 | 0.1444 | -0.2296 | 0.1178 | -0.0608 | 0.275 | 0.8018 |
| AsTot | E2_meter_kit | 16 | 0.035 | 0.3928 | 0.1444 | -0.1608 | 0.1929 | -0.0188 | 0.1999 | 0.5905 |
| AsTot | E3_meter_kit_titration | 8 | 0.0576 | 0.4308 | 0.1953 | -0.1057 | 0.1371 | -0.0408 | 0.2937 | 0.7865 |
| AsTot | E3_meter_kit_titration | 16 | 0.0576 | 0.4308 | 0.1953 | 0.0384 | 0.2513 | 0.0766 | 0.1796 | 0.2724 |
| AsTot | E4_plus_well_record | 8 | 0.104 | 0.476 | 0.2164 | -0.0652 | 0.1617 | -0.0269 | 0.3143 | 0.3784 |
| AsTot | E4_plus_well_record | 16 | 0.104 | 0.476 | 0.2164 | 0.0783 | 0.2693 | 0.0968 | 0.2067 | 0.1983 |
| AsTot | REF_full_cheap | 8 | 0.063 | 0.556 | 0.3611 | -0.038 | 0.2265 | 0.0225 | 0.3295 | 0.42 |
| AsTot | REF_full_cheap | 16 | 0.063 | 0.556 | 0.3611 | 0.0056 | 0.3426 | 0.0552 | 0.2133 | 0.1439 |

## S9. Censoring treatments in full

Four treatments on identical folds, scored against one common reference, with the censored-consistency column that covers the lower tail the common reference excludes.

Table S9. Four censoring treatments on identical folds against one common reference, with censored-consistency.

| target | pct censored | largest detection limit | dl as fraction of median | pct mass at or below dl | n scored against | n censored | consistency substitute half | consistency substitute dl | consistency substitute zero | consistency stochastic imputation | r2 substitute half | r2 substitute dl | r2 substitute zero | r2 stochastic imputation | r2 spread | r2log substitute half | r2log substitute dl | r2log substitute zero | r2log stochastic imputation | r2 log spread | pred shift pct of median |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AsTot | 33.9 | 6 | 0.2727 | 37.3 | 179 | 92 | 46.7 | 27.2 | 59.8 | 42.4 | -0.0308 | 0.0001 | -0.1246 | -0.0468 | 0.1247 | 0.1501 | 0.2164 | -0.0719 | 0.1521 | 0.2883 | 8.4 |
| Ni | 1.1 | 0.4 | 0.1667 | 1.1 | 268 | 3 | 0 | 0 | 0 | 0 | 0.1832 | 0.1814 | 0.1695 | 0.1809 | 0.0137 | 0.272 | 0.2696 | 0.2537 | 0.2692 | 0.0183 | 0.3 |
| Pb | 12.5 | 0.05 | 0.5 | 24.7 | 237 | 34 | 0 | 0 | 2.9 | 0 | -0.0893 | -0.0943 | -0.09 | -0.0927 | 0.0049 | -0.0956 | -0.1015 | -0.0948 | -0.1012 | 0.0067 | 1.4 |
| Zn | 0.4 | 1 | 0.125 | 1.5 | 259 | 1 | 0 | 0 | 0 | 0 | -0.0197 | -0.0164 | -0.0198 | -0.0164 | 0.0034 | 0.0571 | 0.0586 | 0.0572 | 0.0586 | 0.0015 | 0.1 |
| U | 22.9 | 0.01 | 0.0238 | 26.9 | 209 | 62 | 6.5 | 6.5 | 9.7 | 6.5 | 0.2684 | 0.275 | 0.2686 | 0.2684 | 0.0067 | 0.4808 | 0.4839 | 0.4809 | 0.4806 | 0.0033 | 0.2 |
| Mo | 7.4 | 0.1 | 0.0909 | 9.2 | 251 | 20 | 10 | 5 | 25 | 5 | 0.257 | 0.2482 | 0.2483 | 0.2558 | 0.0088 | 0.4251 | 0.4255 | 0.4151 | 0.4295 | 0.0143 | 1.3 |
| Ba | 0 |  |  | 0 | 271 | 0 |  |  |  |  | 0.5255 | 0.5255 | 0.5255 | 0.5255 | 0 | 0.6254 | 0.6254 | 0.6254 | 0.6254 | 0 | 0 |

## S10. Supporting hydrogeochemistry

Table S10. Charge balance error statistics for the analytical quality check.

| statistic | value |
| --- | --- |
| n_evaluated | 271 |
| median_abs_CBE_pct | 1.535 |
| within_5pct | 261 |
| within_10pct | 270 |
| pct_within_10 | 99.6 |

Table S11. Hydrochemical water types by Piper classification.

| water type | n wells | pct wells | median As ugL |
| --- | --- | --- | --- |
| Ca-HCO3 | 182 | 67.2 | 12 |
| Na+K-HCO3 | 40 | 14.8 | 39.5 |
| Mg-HCO3 | 28 | 10.3 | 85 |
| Na+K-Cl | 13 | 4.8 | 22 |
| Mg-Cl | 5 | 1.8 | 6 |
| undetermined | 2 | 0.7 | 47.3 |
| Ca-Cl | 1 | 0.4 | 6 |

Table S12. Arsenic and iron by depth stratum.

| stratum | n wells | median As ugL | mean As ugL | n above WHO | pct above WHO | median Fe mgL |
| --- | --- | --- | --- | --- | --- | --- |
| shallow <20 m | 87 | 65 | 118 | 66 | 75.9 | 1.01 |
| intermediate 20-30 m | 60 | 15 | 259.9 | 33 | 55 | 0.748 |
| deep 30-50 m | 75 | 8.9 | 72.41 | 35 | 46.7 | 1.02 |
| very deep >=50 m | 39 | 6 | 35.61 | 12 | 30.8 | 4.53 |

Table S13. Arsenic by surface geology unit in the national survey.

| GEOL UNIT | n | median | above | pct above |
| --- | --- | --- | --- | --- |
| Chandina Alluvium | 183 | 114 | 126 | 68.9 |
| Deltaic Silt | 428 | 44.85 | 282 | 65.9 |
| Deltaic Sand | 57 | 42.3 | 39 | 68.4 |
| Marsh clay and peat | 345 | 14 | 185 | 53.6 |
| Alluvial silt and clay | 476 | 10 | 237 | 49.8 |
| Tidal deltaic deposits | 352 | 6 | 163 | 46.3 |
| Alluvial Sand | 117 | 6 | 50 | 42.7 |
| Valley aluvium & colluviu | 83 | 6 | 25 | 30.1 |
| Alluvial Silt | 599 | 5.9 | 258 | 43.1 |
| Young gravelly sand | 326 | 2.15 | 94 | 28.8 |
| Estuarine deposits | 68 | 1.35 | 8 | 11.8 |
| Dihing & Dupi Tila undivi | 42 | 0.9 | 6 | 14.3 |
| Barind Clay Residuum | 205 | 0.5 | 0 | 0 |
| Madhupur Clay Residuum | 73 | 0.5 | 0 | 0 |
| Old gravelly sand | 111 | 0.5 | 4 | 3.6 |